Maintenance Cleaning

&

Care Guide

Isn’t it amazing how new windows and doors make such a difference in the look of your home?  Your windows and doors will continue to perform like new if you follow these simple cleaning and maintenance routines:

HOW TO CLEAN AND CARE FOR YOUR WINDOW GLASS & FRAMES  ANN

Your aluminum window fames can continue to look like new, even years from now, with regular care and maintenance. Keeping your windows clean will keep them beautiful.  And don’t forget; insulating, low-e or heat reflective glass requires proper maintenance to continue working effectively.

Cleaning Do’s

- DO clean glass, window and door frames with a mixture of mild dish soap and water.

- DO test cleaners on an inconspicuous area first.

- DO rinse completely with clear water, then wipe dry with a soft cloth.

- DO clean tracks and weep holes.

- DO check weather stripping and hardware.

- DO clean your glass with proper cleaning agents. Use any mild household cleaner (non abrasive) – i.e. soap and water, Windex, Pine Sol, or Vinegar, No Ammonia based Products.

- DO clean screens by first removing, then washing on a flat, clean surface with mild soap and water and a soft brush.  Rinse, wipe dry and reinstall.

Cleaning Don’ts

- DON’T use a razor blade, putty knife or abrasive pad.

- DON’T wash glass in direct sunlight.

- DON’T use any petroleum-based, abrasive or caustic cleaners

or solvents because they might cause permanent damage to the frame finish or the Glass.

- DON’T use a high-pressure spray nozzle when rinsing your windows after washing.

- DON’T use oil-based lubricants or damage weep hole covers/baffles.

- DON’T live with poor performing components.

- DON’T add attachments to window or glass unless approved by the window manufacturer.

HOW TO MAKE SURE YOUR WINDOWS AND DOORS OPEN, CLOSE, LOCK AND UNLOCK EASILY FOR YEARS TO COME

Keep your windows and doors operating smoothly by performing these steps:

- Vacuum dirt from sill and track areas regularly, and before each washing.

- Check to make certain that drainage or weep holes are clear of dirt or obstructions.  Weep holes can be found outside the window or door in the bottom of the frame.  Use a small, soft bottle brush to clear openings. Note: If the window is “stacked”, there may be weep holes between units.

- Moving parts in hardware components and tracks and rollers should be lubricated regularly.  In salt-air environments, this can mean at least monthly.

- Check weather stripping around operable window and door panels to be sure it seals evenly.

- Adjust sliding door rollers for proper height clearances.  Most door rollers can be adjusted with a screwdriver through access holes in either the end or side of the sliding panel at the bottom.  Be sure to lift the panel to take the weight off of the roller during roller adjustment.

- Rolling screen doors can be adjusted to glide smoothly.  Use a screwdriver – often in all four corners – to make adjustment.

After making roller adjustments, it may be necessary to also make adjustments in the lock strike placement.  Most strikes may be adjusted by loosening screw fasteners, moving strike plate and retightening.  Check for proper lock operation.

HOW TO MAINTAIN ALUMINUM WINDOWS IN COASTAL AREAS

If the windows are new and have not yet been exposed to a corrosive environment:

- Remove all stickers and labels (except the Dade County  Product Control Label) immediately by soaking them first with water from a spray bottle.  Remove moistened labels with a soft cloth.  Never use a razor blade, putty knife or abrasive pad to clean your frame or glass.  Wipe dry with a soft cloth.

- Wax your aluminum frames with a high quality car wax.  This is very important in areas within 10 air miles of the ocean because salt-laden air may corrode or stain aluminum surfaces left unprotected.

If windows are already installed and have been exposed to the elements;

- Use mineral spirits to remove any salt deposits, dirt, etc.

- Use a light soap and water solution to wash windows, and rinse well.

- Wax your aluminum frames with a high quality car wax.   In corrosive environments it is recommended that windows be rinsed regularly to reduce salt deposit build-ups and ease the annual cleaning process.

HOW TO REDUCE WINDOW CONDENSATION

How does that water get on the inside of windows? And what can you do about it?

Condensation on the inside of a window is the result of a higher air moisture content contacting lower temperatures on the glass.  The higher the interior humidity and the lower the outside window temperature, the more condensation can occur.

Excessive interior humidity can lead to structural damage and health concerns if high moisture levels are sustained inside wall cavities.  Wood rot, mold and mildew can result.

- Check sashes for smooth adequate operation.

- Use manufacturer-approved exhaust fans, especially when showering.

- Install and use a dehumidifier.

- Use ceiling fans to circulate air.

- Open windows and doors whenever possible to allow interior moisture to escape.

By following these simple steps, you’ll add years to the life of your windows and doors.  They will continue to function like new and  they will keep your home looking beautiful.